BANKS COUNTY EMERGENCY SERVICES

RFP Station 23 Generator, Station 33 Generator,

Station 34 Generator, Pless Pump Station Generator, Station 41 Generator, Station 24 Generator, and Annex Transfer Switch

Advertised: April 22, 2024

Pre-Bid Conference: April 29, 2024

Sealed Bids Due: May 16, 2024

Potential Bid Award:  May 21, 2024

INVITATION TO BID/REPLY

NOTE:  This RFP format must be used for your bid format.  All points within the bid package must be answered as either ‘Understood and/or Compliant’, ‘Non-Compliant’ or ‘Non-Compliant with Alternate Solution Provided’.  If you answer with the last selection, you should provide thorough information on your alternate solution.  Any point not addressed will be considered incomplete and may result in disqualification.

Understood and/or Compliant			Non-Compliant			Non-Compliant/ Alternate Solution Provided

General Instructions:

- This bid is for three (3) turnkey 20kw fixed  generators, one (1) 60 KW fixed generator, two (2) 45KW fixed generator, and one (1) automatic transfer switch.   Located at:

Banks County Fire Station 23 at 104 West Banks Dr. Alto, GA 30510,

Banks County Fire Station 33 at  195 Carson Segars Rd, Maysville, GA 30558,

Banks County Fire Station 34 at 1679 HWY 326, Commerce, GA 30529, and

Banks County Pless Pump Station at 103 Pless Drive, Alto, GA

Banks County Fire Station 411 at 2440 Damascus Rd, Baldwin, Ga 31827

Fire Station 24 at 144 Hickory Flat Rd. Gillsville, Ga 30543

Banks County Annex at 150 Hudson Ridge, Homer Ga 30547

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- The invitation is advertised April 22, 2024.  You should report your desire to bid no later than April 29, 2024.  To inform us of your intent to bid; please email blangston@co.banks.ga.us and call 706-677-1812. We will conduct a pre-bid meeting on April 29, 2024 at 11am at the Banks County Annex located at 150 Hudson Ridge in Homer, Georgia.   To ask any questions; please send an email blangston@co.banks.ga.us.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- All bids must be delivered to the Banks County Commissioners Office which is located inside the Banks County Annex at 150 Hudson Ridge, Homer, Georgia 30547.  You must include 2 copies of your bid and they must be delivered sealed and properly labeled with the following title: RFP Station 23, Station 33, Station 34, Pless Generator, Station 41, Station 24, and Annex Transfer Switch.  It must be delivered in a sealed package, on or before 10:00 o’clock A.M., on May 16, 2024.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- Pricing proposals must be on the Bidder Response Form(s) attached, with explanations, if necessary, on separate identified sheets, and must state the total bid “expected not to exceed” price with a breakdown of price for each option.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- Bids will be opened on May 16, 2024 at 10:00am at the Banks County Annex.  It is not required that Bidder be present as no decision will be made at the opening.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- Bid award will be announced following the approval by the Board of Commissioners.  It is imperative that you ensure that your bid is valid for a minimum time of 90 to 180-days past opening.  This will allow time to review the entire bid package, make install adjustments for any inclement weather and either make award or reissue the RFP.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- Banks County incurs no obligation by extending this invitation and all RFP responses will be received without obligation, and County will not incur any costs associated with submission of a bid.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- Bids will not be refused or rejected for minor errors, but not utilizing the bid forms and responding point by point as requested, will result in “the bid being rejected”.  It is the bidder’s responsibility to seek clarification to any question concerning the bid prior to submission of the bid, from the supervisor above named.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- The County reserves the right to reject any and all bid responses.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- The bid is expected to be awarded to the lowest responsible bidder, considering price, meeting or exceeding specifications, growth potential, flexibility to meet changing technology, redundancy, overall capability, additional features available, and the following elements:

  - Quality of workmanship

  - Equipment performance and reliability

  - Design features advantageous to the user

  - Freedom from premature obsolescence

  - Ease of installation, removal, servicing

  - Equipment interchangeability

  - Safety and security provisions

  - Warranties and service contracts available

  - Minimum Installation time

  - Redundancy

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- Any other terms, costs, conditions, or options that would affect your bid that have not been requested or specified in the RFP package must be noted and included in the submitted bid response.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- All persons working for the winning bidder will conduct him or herself in a professional manner at all times.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- The County reserves the right to make modifications in specifications and/or conditions prior to response opening if deemed necessary, in which event, all bidders will be timely notified, and the time for responses extended. Failure to respond as requested may result in the elimination of your entire response from consideration.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- Each bid responder will submit with its response the following:

  - References, with names, addresses and telephone numbers of past generator projects at emergency services.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- Winning bidder will provide coordination for all phases (this includes coordination and direction from the County Maintenance Director and coordination with any current service providers and pertinent Department representatives).  This will also include any needed and appropriate meetings and any efforts to successfully complete full project.  The ability to effectively work together is tantamount with the success of the project.  The County Maintenance Director and proper Department Head Representative along with the EMA/E911 Director, EMA Deputy Director, and Finance Director will be the primary points of contact for the work.  All are needed as all critical systems will be covered by these generators.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- The vendor will provide a plan to have all equipment delivered, programmed and operational.  It is vital that all progress be communicated to all throughout the process to ensure that plans can be made should problems be encountered.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- The vendor submitting the bid shall be responsible for all aspects of the project. The vendor will designate and identify a Project Manager with whom the County Maintenance Director, 911 Maintenance Provider, proper Department Heads and Director will communicate. The Project Manager shall be available at all times during the project and onsite during work.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- All systems must be grounded.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- The County reserves the right to reject any subcontractors as may be required.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- Reports, plans, data, statistics, specifications, data extracted/converted/exported and other supporting records compiled or prepared in the preparation of the product or in performance of the support/services required by this contract, shall be the absolute property of the County and shall not be used by the Contractor for purposes unrelated to this contract without the prior written approval of the County. Such original documents shall be turned over to the County upon completion of the contract except that Contractor shall have the right to retain copies of the same as needed only for support.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- The County reserves the right to cancel the contract and discontinue the services with a thirty (30) day written notice as a result of the failure of the Contractor to provide acceptable work and services as delineated in the response to this document.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- It is agreed that the Contractor will not assign, transfer, convey, or otherwise dispose of a contract that may result from this bid or his right, title, or interest in or to the same, or any part thereof, without written consent of the County.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- To the fullest extent permitted by law, the Contractor will indemnify, defend, and hold Banks County harmless from and against any and all claims, damages, losses, and expenses, including, but not limited to, fees and charges of attorneys and court and arbitration costs, arising out of or resulting from the negligent acts, negligent omissions, willful misconduct, or reckless misconduct of the Contractor or anyone for whom the Contractor is responsible.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- Bidder Certification and Price forms must be filled out.  A contractor affidavit such as the example shown herein must be provided by the winning bidder.  All forms are located at the end of this document. Failure to fill these out may mean disqualification.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

- Evaluation of proposals will include the following during administrative reviews and will aid in determining recommendation that is made for award:

- Was the proposal submitted by the deadline?

- Was the proposal formatted, answered point by point, sealed and labeled as required?

- Does the bidder display that they will maintain quality of workmanship and excellence in service and support?

- Can they meet the acquisition time?

- Cost?

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

Minimum Expectations:

(Note: Due to the complexity, this may not be all inclusive but gives the bottom-line basics needed. Explain specifically what the generators will cover and answer final compliance question at the very bottom of minimum expectations to reflect compliance for body of these expectations.)

- SUPPORT

  - A system with operational customer service and support that is superb.

  - A system that has a warranty for no less than one (1) year.

  - A system that has maintenance that includes both on-site and off-site assistance as needed.

- LOCATIONS CURRENTLY TO RECEIVE GENERATORS AND BE POWERED BY GENERATORS:

Banks County Fire Station 23 at 104 West Banks Dr. Alto, GA 30510, ( 20KW)

Banks County Fire Station 33 at  195 Carson Segars Rd, Maysville, GA 30558 (20KW),

Banks County Fire Station 34 at 1679 HWY 326, Commerce, GA 30529 (20kw), and

Banks County Pless Pump Station  at 103 Pless Drive, Alto, GA (60KW)

Banks County Fire Station 41 at 2440 Damascus Rd. Baldwin, Ga (45KW)

Banks County Fire Station 24 at 144 Hickory Flat Rd. Gillsville, GA 30543 (45KW)

Banks County Annex at 150 Hudson Ridge. Homer, GA 30547 ( Automatic Transfer Switch)

- FUTURE

  - A system capable of allowing several years of growth at the location, allowing for many additions of equipment that will be protected/powered by the generator in the future.  The Department Head Representative can speak to what the future holds and what will have to be covered.

  - An operational system that is not difficult to use, one that is self-sufficient, one that is not prematurely operationally obsolete and one in which upgrades and regular maintenance are included.

- EXAMPLE SPECIFICATIONS

  - Each generator should crank automatically if there is a loss of line power.

  - Three Fire Stations need to include: Minimum 20 KW, 240/120-volt single phase, propane powered with 24-hour fuel tank with sound attenuated weather enclosure with main breaker and remote annunciator.  Cast in place concrete pad.  Connect new generator to new automatic transfer switch to be provided and connected to existing electrical panel in building

  - Station 41 and Station 24 need to include: Minimum 45 KW, 240/120-volt single phase, diesel powered with 24-hour fuel tank with sound attenuated weather enclosure with main breaker and remote annunciator. Cast place concrete pad. Connect new generator to new automatic transfer switch to be provided and connected to existing electrical panel in building.

  - Pless Rd needs to include: Minimum 60KW, 277/480- volt three phase, diesel powered with 24-hour sub-base fuel tank with sound attenuated weather enclosure with main breaker and remote annunciator. Cast in place concrete pad. Connect new generator to new automatic transfer switch to be provided and connected to current electrical panel in building.

  - Banks County Annex needs to include: new automatic transfer switch to be provided and connected to existing electrical panel in building.  Connection to be provided near or withing 25 ft of parking lot and accessible by a portable generator.

  - Any other need to make generator on-line and ready.

  - Customer desires auto-start and stop and automatic weekly testing to ensure the critical facility is always ready.

- REPORTS

  - The system should track all usage and be able to provide reports on testing if needed.

This serves as acknowledgement that all minimum requirements listed above have been reviewed and can be met and method of accomplishment is explained.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

SCOPE OF THE PROJECT:

The vendor will be responsible for providing a turn-key solution for two site generators.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

ACCEPTANCE:

Payment will be made at the rates of 50% on award, 50% on completion and final acceptance following satisfactory performance.

Understood and/or Compliant			Non-Compliant			Non-Compliant/Alternate Solution Provided

BIDDER’S CERTIFICATION

RFP Station 23, Station 33, Station 34, Pless Water, Station 41, Station 24 Generators and Annex Transfer Switch

I certify that this Proposal is submitted without prior understanding, agreement or connection with any corporation, firm or person submitting a Proposal for the same goods/services and is in all respects fair and without collusion or fraud. I understand that collusive bidding is a violation of state and Federal law and can result in fines, prison sentences and civil damages awards. I agree to abide by all conditions of this bid and certify that I am authorized to sign this bid for the bidder.

Bidder Information Name and Mailing Address

(Type or Print) (Where to Send Payment)

Name of Company

Address Address

City, State, & Zip Code

Phone Number

Fax # 							Email Address

__________________________ 	   OR	__________________________

Tax ID Number 				Social Security Number

Name & Title of Person Authorized to Sign

Name 							SIGNATURE

Title

Proposals or Bids not signed shall be declared as “Non-Responsive”

and may not be considered for award.

EXAMPLE OF BIDDER’S / CONTRACTOR AFFIDAVIT

BIDDER PRICE RESPONSE FORM:

RFP Station 23 Generator

Provide details on an additional sheet if necessary.

- Total Cost :									___________________

- Total Reoccurring Cost (Maintenance etc):						___________________

- Other pricing (hardware, options or otherwise) described:				___________________

Attach additional if needed.

BIDDER PRICE RESPONSE FORM:

RFP Station 33 Generator

Provide details on an additional sheet if necessary.

- Total Cost :									___________________

- Total Reoccurring Cost (Maintenance etc):						___________________

- Other pricing (hardware, options or otherwise) described:				___________________

Attach additional if needed.

BIDDER PRICE RESPONSE FORM:

RFP Station 34 Generator

Provide details on an additional sheet if necessary.

- Total Cost :									___________________

- Total Reoccurring Cost (Maintenance etc):						___________________

- Other pricing (hardware, options or otherwise) described:				___________________

Attach additional if needed.

BIDDER PRICE RESPONSE FORM:

RFP Station 41 Generator

Provide details on an additional sheet if necessary.

- Total Cost :									___________________

- Total Reoccurring Cost (Maintenance etc):						___________________

- Other pricing (hardware, options or otherwise) described:				___________________

Attach additional if needed.

BIDDER PRICE RESPONSE FORM:

RFP Station 24 Generator

Provide details on an additional sheet if necessary.

- Total Cost :									___________________

- Total Reoccurring Cost (Maintenance etc):						___________________

- Other pricing (hardware, options or otherwise) described:				___________________

Attach additional if needed.

BIDDER PRICE RESPONSE FORM:

RFP Pless Pump Station Generator

Provide details on an additional sheet if necessary.

- Total Cost :									___________________

- Total Reoccurring Cost (Maintenance etc):						___________________

- Other pricing (hardware, options or otherwise) described:				___________________

Attach additional if needed.

BIDDER PRICE RESPONSE FORM:

RFP Annex Transfer Switch

Provide details on an additional sheet if necessary.

- Total Cost :									___________________

- Total Reoccurring Cost (Maintenance etc):						___________________

- Other pricing (hardware, options or otherwise) described:				___________________

Attach additional if needed.

END OF DOCUMENT